Auto APN Feature

Operator Identification using IMSI (AT+CIMI)

The operator will be identified using the IMSI (International Mobile Subscriber Identity) obtained using the AT+CIMI command.

Send:

AT+CIMI<CR><LF>

Example Response:

AT+CIMI

404450625675571

OK

The IMSI is parsed as follows:

Example:
IMSI = 404450625675571
↓
MCC = 404
MNC = 45
↓
Operator = Airtel

The extracted MCC-MNC is compared against an internal lookup table to determine the corresponding operator and APN. Since the IMSI represents the SIM home network, the MCC/MNC remains unchanged even when the device is roaming. Therefore, AT+CIMI provides a more reliable mechanism for Auto APN selection than using the currently attached network.

MCC-MNC Mapping

| MCC-MNC | Operator | APN |
| --- | --- | --- |
| 404-10 | Airtel | senseltele |
| 404-45 | Airtel | senseltele |
| 404-49 | Airtel | senseltele |
| 404-84 | Vi | M2MISAFE |
| 404-86 | Vi | M2MISAFE |
| 405-840 | Jio | jionet / Company APN |
| 404-80 | BSNL | bsnlnet / Company APN |
| International | International SIM | User Configured / Default APN |

APN Assignment

The APN will be configured only once during device power-on. Since the SIM is expected to remain unchanged during normal operation, performing operator detection on every wake-up would introduce unnecessary AT command exchanges, increase wake-up time, and slightly increase power consumption. Therefore, the APN is determined during power-up and stored in PmData.Apn for subsequent data connections. If the SIM is replaced, a manual reset can be introduced to refresh the APN.

Fallback Mechanism

If operator detection fails or the detected MCC-MNC is not available in the internal lookup table, the firmware will use a fallback mechanism.

Read IMSI → Extract MCC/MNC → Lookup APN Table → Entry Found?
Yes → Configure APN
No → Use Default APN.

This ensures that the device can continue attempting data connectivity even if operator identification is unsuccessful or the operator is not available in the supported APN list.

Manual APN Configuration

The existing SET APN command will continue to be used for manual APN configuration.

When a user configures the APN manually using the SET APN command, the manually configured APN will be stored in the device along with a Manual APN Enable flag.

During device power-up, the firmware will first check whether a Manual APN has been configured.

• If Manual APN is enabled, the firmware will directly copy the manually configured APN into PmData.Apn and skip the Auto APN detection process.
• If Manual APN is not enabled, the firmware will perform Auto APN detection using the IMSI (AT+CIMI), identify the operator based on the MCC-MNC lookup table, and configure the corresponding APN.

This ensures that a manually configured APN is never overwritten by the Auto APN mechanism.

APN Selection Priority:
1. Manual APN (Highest Priority)
2. Auto APN using IMSI lookup table
3. Default/Previous APN (Fallback)

Once a Manual APN is configured, the firmware will continue using it until the user clears the Manual APN configuration or re-enables Auto APN mode.

| MCC-MNC | Operator | APN |
| --- | --- | --- |
| 404-01 | Vi | M2MISAFE |
| 404-02 | Airtel | senseltele |
| 404-03 | Airtel | senseltele |
| 404-04 | Vi | M2MISAFE |
| 404-05 | Vi | M2MISAFE |
| 404-07 | Vi | M2MISAFE |
| 404-09 | Reliance | Company APN |
| 404-10 | Airtel | senseltele |
| 404-11 | Vi | M2MISAFE |
| 404-12 | Idea | M2MISAFE |
| 404-13 | Vi | M2MISAFE |
| 404-14 | Vi | M2MISAFE |
| 404-15 | Vi | M2MISAFE |
| 404-16 | Airtel | senseltele |
| 404-17 | Airtel | senseltele |
| 404-18 | Reliance | Company APN |
| 404-19 | Idea | M2MISAFE |
| 404-20 | Vi | M2MISAFE |
| 404-21 | Loop | Default APN |
| 404-22 | Idea | M2MISAFE |
| 404-24 | Idea | M2MISAFE |
| 404-27 | Vi | M2MISAFE |
| 404-28 | Airtel | senseltele |
| 404-29 | Airtel | senseltele |
| 404-30 | Vi | M2MISAFE |
| 404-31 | Airtel | senseltele |
| 404-34 | BSNL | bsnlnet / Company APN |
| 404-36 | Reliance | Company APN |
| 404-38 | BSNL | bsnlnet / Company APN |
| 404-40 | Airtel | senseltele |
| 404-41 | Airtel | senseltele |
| 404-42 | Airtel | senseltele |
| 404-43 | Vi | M2MISAFE |
| 404-44 | Vi | M2MISAFE |
| 404-45 | Airtel | senseltele |
| 404-46 | Vi | M2MISAFE |
| 404-47 | Vi | M2MISAFE |
| 404-49 | Airtel | senseltele |
| 404-50 | Reliance | Company APN |
| 404-51 | BSNL | bsnlnet / Company APN |
| 404-52 | Reliance | Company APN |
| 404-53 | BSNL | bsnlnet / Company APN |
| 404-54 | BSNL | bsnlnet / Company APN |
| 404-55 | BSNL | bsnlnet / Company APN |
| 404-56 | Idea | M2MISAFE |
| 404-57 | BSNL | bsnlnet / Company APN |
| 404-58 | BSNL | bsnlnet / Company APN |
| 404-59 | BSNL | bsnlnet / Company APN |
| 404-60 | Vi | M2MISAFE |
| 404-62 | BSNL | bsnlnet / Company APN |
| 404-64 | BSNL | bsnlnet / Company APN |
| 404-66 | BSNL | bsnlnet / Company APN |
| 404-68 | MTNL | Default APN |
| 404-70 | Airtel | senseltele |
| 404-71 | BSNL | bsnlnet / Company APN |
| 404-72 | BSNL | bsnlnet / Company APN |
| 404-73 | BSNL | bsnlnet / Company APN |
| 404-74 | BSNL | bsnlnet / Company APN |
| 404-75 | BSNL | bsnlnet / Company APN |
| 404-76 | BSNL | bsnlnet / Company APN |
| 404-77 | BSNL | bsnlnet / Company APN |
| 404-78 | Vi | M2MISAFE |
| 404-79 | CellOne | Default APN |
| 404-80 | BSNL | bsnlnet / Company APN |
| 404-81 | BSNL | bsnlnet / Company APN |
| 404-82 | Idea | M2MISAFE |
| 404-84 | Vi | M2MISAFE |
| 404-86 | Vi | M2MISAFE |
| 405-025 | Tata Docomo | Default APN |
| 405-026 | Tata Docomo | Default APN |
| 405-027 | Tata Docomo | Default APN |
| 405-028 | Aircel | Default APN |
| 405-029 | Aircel | Default APN |
| 405-030 | Tata Docomo | Default APN |
| 405-031 | BSNL | bsnlnet / Company APN |
| 405-032 | BSNL | bsnlnet / Company APN |
| 405-033 | BSNL | bsnlnet / Company APN |
| 405-034 | BSNL | bsnlnet / Company APN |
| 405-035 | Aircel | Default APN |
| 405-840 | Jio | jionet / Company APN |
| International | International SIM | User Configured / Default APN |